Tender Specification Bipolar Isolation Amplifier DB 64000

Place the cursor in your Web browser's address bar and scan the bar code. You find directly into the right service web portal. 
DRAGO simple – reliable

Bipolar Isolation Amplifier, Zero/Span compensation, calibrated signal setting

Isolation and conversion of bipolar and unipolar industrial signals,

slim housing for DIN-rail mounting, width 6.2 mm, protection class: IP 20

Input				± 20 mA, ± 10 mA, 0 - 20 mA, 4 - 20 mA, 0 - 10 mA, 2 - 10 mA
				± 10 V, ± 5 V, 0 - 10 V, 2 - 10 V, 0 - 5 V, 1 - 5 V
Output				± 20 mA, ± 10 mA, 0 - 20 mA, 4 - 20 mA, 0 - 10 mA, 2 - 10 mA
				± 10 V, ± 5 V, 0 - 10 V, 2 - 10 V, 0 - 5 V, 1 - 5 V
Range setting			calibrated signal setting

Subsequent readjustment	zero/span potentiometers on the front panel, switchable

Cut-off frequency		8 kHz, switchable to 100 Hz

Transmission error		< 0.1 % full scale

Power supply			9.6 V to 31.2 V DC, approx. 0.8 W, In-Rail-Bus

Protective separation		EN 61140, 3-Port isolation up to 300 V AC/DC between all circuits

Test voltage			3 kV, 50 Hz, 1 min., input against output against power supply

Working voltage		600 V AC/DC for overvoltage category II and pollution degree 2

Type				Bipolar Isolation Amplifier DB 64000 ___	(see order code)

Supplier			DRAGO Automation GmbH, Berlin, Germany
				Phone: +49-(0)30-409982-0, Fax: +49-(0)30-409982-10, 
				email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent

Order code:

| Bipolar Isolation Amplifier, calibrated range selection | DB 64000 S |
| --- | --- |
| Bipolar Isolation Amplifier, calibrated range selection, In-Rail-Bus for power supply | DB 64000 B |