Tender Specification Isolation Amplifier Module DN 240M

Place the cursor in your Web browser's address bar and scan the bar code. You find directly into the right service web portal. 
DRAGO simple – reliable

Isolation Amplifier Module, terminal selectable

Isolation and Conversion of Standard Signals for PCB Assembling,

Module for pcb assembling, 52.5 x 36 x 15.5 mm

Input			0 - 20 mA, 4 - 20 mA, 0 - 10 V 
Output			0 - 20 mA, 4 - 20 mA, 0 - 10 V 
Range setting		terminal selectable, calibrated signal setting

Cut-off frequency	1 kHz

Transmission error	< 0.3 % full scale
Temperature coeff.	150 ppm/K of final value

Power Supply		24 V DC, ± 10 %, approx. 1.2 W

Test voltage		2.5 kV, 50 Hz, 1 min., input against output against power supply

Working voltage	600 V AC/DC for overvoltage category II and pollution degree 2

Type			Isolation Amplifier DN 240M

Supplier		DRAGO Automation GmbH, Berlin, Germany

Phone: +49-(0)30-409982-0, Fax: +49-(0)30-409982-10,

email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent