Tender Specification Isolation Amplifier DN 2400

Place the cursor in your Web browser's address bar and scan the bar code. You find directly into the right service web portal. 
DRAGO simple – reliable

Isolation Amplifier, calibrated signal setting

Isolation and conversion of standard signals in low-cost applications,

slim housing for DIN-rail mounting, width 12.5 mm, protection class: IP 20

Input			0 - 20 mA, 4 - 20 mA, 0 - 10 V

Output			0 - 20 mA, 4 - 20 mA, 0 - 10 V

Range setting		simple selectable via DIP switches

Cut-off frequency	1 kHz

Transmission error	< 0.3 % full scale

Power Supply		Universal power supply for 20 - 253 V AC/DC

Isolation		3-port isolation

Test voltage		2.5 kV, 50 Hz, 1 min., input against output against power supply

Working voltage	600 V AC/DC for overvoltage category II and pollution degree 2

Type			Isolation Amplifier DN 2400 AG

Supplier		DRAGO Automation GmbH, Berlin, Germany

Phone: +49-(0)30-409982-0, Fax: +49-(0)30-409982-10,

email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent