Tender Specification Isolation Amplifier DN 28

Place the cursor in your Web browser's address bar and scan the bar code. You find directly into the right service web portal. 
DRAGO simple – reliable

8 versions available:

Isolation Amplifier, fixed setting: 0 - 20 mA to 0 - 20 mA, or 4 - 20 mA to 4 - 20 mA

Cost optimized isolation of standard signals,

slim housing for DIN-rail mounting, installation depth 60 mm, width 11.2 mm, protection class: IP 20

Input			0(4) - 20 mA 
Output			0(4) - 20 mA 
Transmission error	< 0.2 % fullscale

Operation temeprature	–20 °C to +60 °C   (–4 °F to +140 °F)

Power supply		24 V AC/DC

Protective separation	EN 61140, 3-Port isolation up to 300 V AC/DC between all circuits

Test voltage		3 kV, 50 Hz, 1 min., input against output against power supply

Working voltage	600 V AC/DC for overvoltage category II and pollution degree 2

Type			Isolation Amplifier DN 28 P–12

Supplier		DRAGO Automation GmbH, Berlin, Germany

Phone: +49-30-409982-0, Fax: +49-30-409982-10,

email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent

Isolation Amplifier, fixed setting: 4 - 20 mA to 0 - 20 mA

Cost optimized isolation of standard signals,

slim housing for DIN-rail mounting, installation depth 60 mm, width 11.2 mm, protection class: IP 20

Input			4 - 20 mA 
Output			0 - 20 mA 
Transmission error	< 0.2 % full scale

Operation temeprature	–20 °C to +60 °C   (–4 °F to +140 °F)

Power supply		24 V AC/DC

Protective separation	EN 61140, 3-Port isolation up to 300 V AC/DC between all circuits

Test voltage		3 kV, 50 Hz, 1 min., input against output against power supply

Working voltage	600 V AC/DC for overvoltage category II and pollution degree 2

Type			Isolation Amplifier DN 28 P–32

Supplier		DRAGO Automation GmbH, Berlin, Germany

Phone: +49-30-409982-0, Fax: +49-30-409982-10,

email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent

Isolation Amplifier, fixed setting: 0 - 10 V to 0 - 20 mA

Cost optimized isolation of standard signals,

slim housing for DIN-rail mounting, installation depth 60 mm, width 11.2 mm, protection class: IP 20

Input			0 - 10 V 
Output			4 - 20 mA 
Transmission error	< 0.2 % full scale

Operation temeprature	–20 °C to +60 °C   (–4 °F to +140 °F)

Power supply		24 V AC/DC

Protective separation	EN 61140, 3-Port isolation up to 300 V AC/DC between all circuits

Test voltage		3 kV, 50 Hz, 1 min., input against output against power supply

Working voltage	600 V AC/DC for overvoltage category II and pollution degree 2

Type			Isolation Amplifier DN 28 P–52

Supplier		DRAGO Automation GmbH, Berlin, Germany

Phone: +49-30-409982-0, Fax: +49-30-409982-10,

email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent

Isolation Amplifier, fixed setting: 0 - 20 mA to 4 - 20 mA

Cost optimized isolation of standard signals,

slim housing for DIN-rail mounting, installation depth 60 mm, width 11.2 mm, protection class: IP 20

Input			0 - 20 mA 
Output			4 - 20 mA 
Transmission error	< 0.2 % full scale

Operation temeprature	–20 °C to +60 °C   (–4 °F to +140 °F)

Power supply		24 V AC/DC

Protective separation	EN 61140, 3-Port isolation up to 300 V AC/DC between all circuits

Test voltage		3 kV, 50 Hz, 1 min., input against output against power supply

Working voltage	600 V AC/DC for overvoltage category II and pollution degree 2

Type			Isolation Amplifier DN 28 P–14

Supplier		DRAGO Automation GmbH, Berlin, Germany

Phone: +49-30-409982-0, Fax: +49-30-409982-10,

email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent

Isolation Amplifier, fixed setting: 0 - 10 V to 4 - 20 mA

Cost optimized isolation of standard signals,

slim housing for DIN-rail mounting, installation depth 60 mm, width 11.2 mm, protection class: IP 20

Input			0 - 10 V 
Output			4 - 20 mA 
Transmission error	< 0.2 % full scale

Operation temeprature	–20 °C to +60 °C   (–4 °F to +140 °F)

Power supply		24 V AC/DC

Protective separation	EN 61140, 3-Port isolation up to 300 V AC/DC between all circuits

Test voltage		3 kV, 50 Hz, 1 min., input against output against power supply

Working voltage	600 V AC/DC for overvoltage category II and pollution degree 2

Type			Isolation Amplifier DN 28 P–54

Supplier		DRAGO Automation GmbH, Berlin, Germany

Phone: +49-30-409982-0, Fax: +49-30-409982-10,

email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent

Isolation Amplifier, fixed setting: 0 - 20 mA to 0 - 10 V

Cost optimized isolation of standard signals,

slim housing for DIN-rail mounting, installation depth 60 mm, width 11.2 mm, protection class: IP 20

Input			0 - 20 mA 
Output			0 - 10 V 
Transmission error	< 0.2 % full scale

Operation temeprature	–20 °C to +60 °C   (–4 °F to +140 °F)

Power supply		24 V AC/DC

Protective separation	EN 61140, 3-Port isolation up to 300 V AC/DC between all circuits

Test voltage		3 kV, 50 Hz, 1 min., input against output against power supply

Working voltage	600 V AC/DC for overvoltage category II and pollution degree 2

Type			Isolation Amplifier DN 28 P–16

Supplier		DRAGO Automation GmbH, Berlin, Germany

Phone: +49-30-409982-0, Fax: +49-30-409982-10,

email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent

Isolation Amplifier, fixed setting: 4 - 20 mA to 0 - 10 V

Cost optimized isolation of standard signals,

slim housing for DIN-rail mounting, installation depth 60 mm, width 11.2 mm, protection class: IP 20

Input			4 - 20 mA 
Output			0 - 10 V 
Transmission error	< 0.2 % full scale

Operation temeprature	–20 °C to +60 °C   (–4 °F to +140 °F)

Power supply		24 V AC/DC

Protective separation	EN 61140, 3-Port isolation up to 300 V AC/DC between all circuits

Test voltage		3 kV, 50 Hz input against output against power supply

Working voltage	600 V AC/DC for overvoltage category II and pollution degree 2

Type			Isolation Amplifier DN 28 P–36

Supplier		DRAGO Automation GmbH, Berlin, Germany

Phone: +49-30-409982-0, Fax: +49-30-409982-10,

email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent

Isolation Amplifier, fixed setting: 0 - 10 V to 0 - 10 V

Cost optimized isolation of standard signals,

slim housing for DIN-rail mounting, installation depth 60 mm, width 11.2 mm, protection class: IP 20

Input			0 - 10 V 
Output			0 - 10 V
Transmission error	< 0.2 % full scale

Operation temeprature	–20 °C to +60 °C   (–4 °F to +140 °F)

Power supply		24 V AC/DC

Protective separation	EN 61140, 3-Port isolation up to 300 V AC/DC between all circuits

Test voltage		3 kV, 50 Hz input against output against power supply

Working voltage	600 V AC/DC for overvoltage category II and pollution degree 2

Type			Isolation Amplifier DN 28 P–56

Supplier		DRAGO Automation GmbH, Berlin, Germany

Phone: +49-30-409982-0, Fax: +49-30-409982-10,

email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent