Tender Specification Resistance Transmitter DR 41

Place the cursor in your Web browser's address bar and scan the bar code. You find directly into the right service web portal. 
DRAGO simple – reliable

Resistance Transmitter, fixed ranges

Measuring of Resistors with Fixed Setting,

slim housing for DIN-rail mounting, installation depth 60 mm, width 11.2 mm, protection class: IP 20

Input signal		Fixed ranges within 20 Ω to 1 MΩ
			Sensor connection: 4-wire, 3-wire, 2-wire	see order code

Output			0 - 20 mA, 4 - 20 mA, 0 - 5 V, 1 - 5 V, 0 - 10 V, 2 - 10 V, see order code

Transmission error	< 0.2 % full scale

Zero/Span komp.	± 5 %

Response time		< 2 ms

Operation temeprature	–20 °C to +60 °C   (–4 °F to +140 °F)

Power supply		24 V AC/DC

Protective separation	EN 61140, 3-Port isolation up to 300 V AC/DC between all circuits

Test voltage		3 kV, 50 Hz, 1 min., input against output against power supply

Working voltage	600 V AC/DC for overvoltage category II and pollution degree 2

Type			Resistance Transmitter DR 41 P – __  __  __   (see order code)

Supplier		DRAGO Automation GmbH, Berlin, Germany

Phone: +49-30-409982-0, Fax: +49-30-409982-10,

email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent

Order code:

|  |  |  | Input | Output |
| --- | --- | --- | --- | --- |
| 2-wire connection DR 41 P – 2 |  |  | X | X |
| 3-wire connection DR 41 P – 3 |  |  | X | X |
| 4-wire connection DR 41 P – 4 |  |  | X | X |
| Input | 0 to 20 Ω |  | 2 |  |
|  | 0 to 50 Ω |  | 3 |  |
|  | 0 to 100 Ω |  | 4 |  |
|  | 0 to 200 Ω |  | 5 |  |
|  | 0 to 500 Ω |  | 6 |  |
|  | 0 to 1000 Ω |  | 7 |  |
|  | 0 to 2000 Ω |  | 8 |  |
|  | 0 to 5000 Ω |  | 9 |  |
|  | 0 to 10 kΩ |  | A |  |
|  | 0 to 20 kΩ |  | B |  |
|  | 0 to 50 kΩ |  | C |  |
|  | 0 to 100 kΩ |  | D |  |
|  | 0 to 200 kΩ |  | E |  |
|  | 0 to 500 kΩ |  | F |  |
|  | 0 to 1 MΩ |  | G |  |
| Output | 0 to 20 mA |  |  | 2 |
|  | 4 to 20 mA |  |  | 4 |
|  | 0 to 5 V |  |  | 5 |
|  | 1 to 5 V |  |  | 8 |
|  | 0 to 10 V |  |  | 6 |
|  | 2 to 10 V |  |  | 7 |