Tender Specification Temperature Transmitter DR 4700

Place the cursor in your Web browser's address bar and scan the bar code. You find directly into the right service web portal. 
DRAGO simple – reliable

Temperature Transmitter, configurable

Temperature measuring with Pt 100 / Pt 1000 sensors,

slim housing for DIN-rail mounting, width 12.5 mm, protection class: IP 20

Input signal			Pt 100 / Pt 1000, switchable

Sensor connection: 4-wire, 3-wire, 2-wire

Measuring zero: -100 °C, -50 °C, 0 °C, 50 °C, switchable

Measuring span: 50 K, 100 K, 200 K, 300 K, switchable

Output				0 - 20 mA, 4 - 20 mA, 0 - 5 V, 1 - 5 V, 0 - 10 V, 2 - 10 V, switchable

Zero/Span potentiometer	zusätzliche Messbereichseinstellung: Zero 0 … 50 °C, Span: 100 … 200 %
Linearity			< 0.2 % of measuring span

Response time			20 ms

Power Supply			Universal power supply for 20 - 253 V AC/DC

Protective separation		EN 61140, 3-Port isolation up to 300 V AC/DC between all circuits

Test voltage			4 kV, 50 Hz, 1 min., input against output against power supply

Working voltage		600 V AC/DC for overvoltage category II and pollution degree 2

Type				Temperature Transmitter DR 4700

Supplier			DRAGO Automation GmbH, Berlin, Germany

Phone: +49-(0)30-409982-0, Fax: +49-(0)30-409982-10,

email: info@drago-automation.de, internet: www.drago-automation.de

or equivalent